Lab Problem Statement — Unit: Spatial Filtering

Restoring and Enhancing Degraded Onboard Camera Feed for an Autonomous Ground Vehicle (AGV)

Context An Autonomous Ground Vehicle (AGV) operating in an unstructured outdoor environment (dust, low light, vibration-induced blur, sensor grain) relies on its onboard monocular camera feed for obstacle detection and terrain classification. Before any high-level vision task (edge-based obstacle boundary detection, terrain segmentation) can run reliably, the raw frame must be pre-processed: noise arising from sensor grain and atmospheric particulates must be suppressed, while object boundaries relevant for navigation must be preserved or enhanced. You are tasked with designing and evaluating a spatial-filtering pre-processing pipeline for such a feed, using only your own from-scratch implementations of the operations covered in class.

# Learning Objectives

- Implement 2D spatial correlation and convolution from first principles (no cv2.filter2D / scipy.signal shortcuts).

- Apply and tune a smoothing (averaging) filter to suppress sensor/atmospheric noise without destroying navigation-relevant edges.

- Apply Laplacian-based sharpening to recover/enhance obstacle boundary information degraded by motion blur.

- Apply unsharp masking and high-boost filtering, and reason about the boost factor k as a tunable design parameter.

- Quantitatively justify filter and parameter choices using objective image quality metrics, not visual impression alone.

# Test Data You Must Generate

You will not be given a pre-cleaned dataset. Part of the exercise is producing a realistic degraded input, exactly as you would have to do when validating a perception pipeline before field trials.

- Acquire or select one grayscale image that plausibly resembles an AGV forward-camera frame (an outdoor scene with clear object boundaries — a path/road with obstacles, a corridor, a parking area, or similar). A personal photo converted to grayscale is acceptable.

- Programmatically degrade this clean image to simulate realistic sensor conditions by adding:

- Gaussian noise (simulating sensor/thermal grain) at two levels: σ = 10 and σ = 25 (on a 0–255 scale).

- Mild motion blur (simulating vehicle vibration) using a linear motion-blur kernel of length 7–9 pixels.

- Keep the original clean image as your ground truth for quantitative comparison in Task 4.

# Tasks

Task 1 — Noise Suppression via Averaging

- Implement a box/averaging filter from scratch (your own correlate2d, not a library call).

- Apply it to both noisy variants (σ = 10 and σ = 25) at three kernel sizes: 3×3, 5×5, 9×9.

- Present all outputs in a labeled grid and describe, in your own words, the trade-off you observe between noise suppression and edge/detail loss as kernel size increases.

Task 2 — Boundary Recovery via Laplacian Sharpening

- Apply your Laplacian sharpening implementation to the motion-blurred image.

- Compare the 4-neighbor Laplacian kernel against the 8-neighbor (diagonals included) variant — implement both and show both Laplacian response maps side by side.

- State which variant recovers obstacle edges more effectively for this image, and justify why, referencing what each kernel is mathematically sensitive to.

Task 3 — Unsharp Masking / High-Boost Design

- Apply unsharp masking (k = 1) and high-boost filtering at k = 1.5, 2, 3 on the noisy image from Task 1 (use the σ = 10 case).

- Plot output sharpness (see metrics below) as a function of k, and identify the point beyond which increasing k stops helping — i.e., where amplified noise outweighs sharpening gain. This is a genuine design decision, not a fixed answer.

Task 4 — Objective Evaluation (Not Just Visual Judgment)

A perception pipeline cannot be tuned by eye alone in a real deployment. For every processed output above, compute:

- PSNR (Peak Signal-to-Noise Ratio) against the clean ground-truth image.

- A simple sharpness/edge-strength metric of your choice (e.g., variance of the Laplacian, or mean gradient magnitude) — define your metric explicitly in your report.

Tabulate PSNR and your sharpness metric for every configuration tested (kernel size × noise level, k value, Laplacian variant). Use this table, not intuition, to recommend a single best pre-processing configuration for this AGV use case.

Task 5 — Pipeline Recommendation

Based on Tasks 1–4, propose and justify a complete two-stage pipeline (denoise → sharpen) with specific parameter values you would actually deploy on the AGV. Your justification must reference your Task 4 numbers, not be a restatement of theory.

# Deliverables

| Deliverable | Format |
| --- | --- |
| Commented source code (all filters from scratch) | .py file |
| Degraded test images (clean + noisy + blurred variants) | .png / .jpg |
| Output grids for Tasks 1–3 | Figures in report |
| PSNR / sharpness metric table (Task 4) | Table in report |
| Written report with pipeline recommendation (Task 5) | PDF, 4–6 pages |

# Evaluation Rubric

| Criterion | Weight |
| --- | --- |
| Correctness of from-scratch filter implementations | 25% |
| Correct, realistic simulation of noise/blur degradation | 10% |
| Depth of analysis in Tasks 1–3 (not just images shown) | 25% |
| Correct and meaningful use of PSNR / sharpness metrics | 20% |
| Quality and justification of final pipeline recommendation | 20% |

# Reflection Questions (Include Written Answers in Your Report)

These are intended to be genuinely debatable — answer with reasoning, not a one-line fact.

- If you had swapped the order of operations in your pipeline — sharpen first, then denoise — would Task 4's numbers change? Try it and report what actually happens, and explain why the order matters (or doesn't) for linear filters.

- Your Laplacian sharpening uses correlation internally. Would using true convolution (flipped kernel) change your Task 2 output at all? Justify using the specific kernel you used — don't just cite the general rule.

- PSNR improved after averaging in Task 1, but a human observer might judge the 9×9 result as 'worse' for navigation. Explain this apparent contradiction — what is PSNR actually measuring, and why might it disagree with task-relevant quality?

- At what noise level (try pushing σ higher than 25) does no amount of sharpening in Task 3 recover usable edges? What does this tell you about the fundamental limit of spatial-domain pre-processing, and what would you need to add to the pipeline beyond simple filtering?

Submission Note This is deliberately open-ended: there is no single 'correct' kernel size or k value. Your grade depends on whether your chosen configuration is defensible using your own Task 4 data, not on matching a reference answer.